- Use SPI interface to communicate with? Just to let things going.

SPI still need checking?

It uses PMU_SPI/SPI3(when normal):

| GPIO_P10 | (i/o)GPIO |  | (i)PMUSPI_CS | (i)DAI_RST | (i/o)I2S_WS |
| --- | --- | --- | --- | --- | --- |
| GPIO_P11 | (i/o)GPIO |  | (i)PMUSPI_DI | (i)DAI_DI | (i)I2S_DI (0) |
| GPIO_P12 | (i/o)GPIO |  | (o)PMUSPI_DO | (o)DAI_DO | (o)I2S_DO |
| GPIO_P13 | (i/o)GPIO |  | (i)PMUSPI_CK | (o)DAI_CLK | (i/o)I2S_BCK |

PMU_SPI_CLK                 : out std_logic                     ;

PMU_SPI_CS                  : out std_logic                     ;

PMU_SPI_DO                  : in std_logic                     ;

PMU_SPI_DI                  : out std_logic

Direction is at PMU view!

When analog_tst is 1, or gpio configured. We can use gpio p10/1/2(out)/3 to go as a SPI. But analog tst is configured by registers. So, go another way: I2C.

I2C needs PMU component address? Like this:

chip _addr_i2c  =  {101  [i2c_addr[1]]  11  [i2c_addr[0]]}  (i2c_addr[2] is not used)

fm, bt, abb address is not changed?!

- Try to access (actually read) registers from PMU interface. But, for all registers, read values are 7FFF_FFFF or BFFF_FFFF.

1. Clarify the wasequence of reading: Start -> chip id(0) -> ADDR(specifying reading register) -> ACK  -> restart -> chip id(1) -> Data in -> NO ACK -> STOP. Waveform ok, No such thing.

2. Try to write, then read back. But once 00 is written, every registers read ok.

Why ? write trigger something? Because write is scan mode?

Cause Power Way is different?!

- Debug Host not having reset detected(mean not released)

Try to look at 32K in, it is 32k for rtc, not for bb(to analog)

2014.02.08

- no 0xFF?!

PMU interface(rda5855 PMU in schematic) accepts POWERKEY, NO matter always on or pulse on, AC_ON(when AC is ok). OR power_bb from software(wakeup). Powered on is continued by software power_bb, down is controlled by Power_bb, too.

Output POWERKEY_BB, that KEYON for dig.

As AC ON keeps on, while PWRKEY is pulse. If AC ON could let 26M on, then AC ON long enough should be OK, too. But, not work. So, I let AC ON, after 26M is on, use reset! Debug Host acknowledges 0xFF.  Time was not under my consideration!

2014.02.10

1. Try to communicate with PMU interface with SPI protocol.

Done. Because software not displays good, but waveform is ok, compared to 8809! Cause a resistance on board! (Also CS should be used!)

2. Find out: why reset in, 26m could be out! As iomux_abb_pmu_tst is not ‘1’ when reset. It seems that OK(software on). But why force on not ok?

Why ALL possibilities not work?!

Because of voltage?

NOT. As with UART tool to check it, things are doing. So, hardware not has such problems. Nop, NO Frame Format.

Wait enough time?  ABB, PMU reading would not toggle very much. OK for debugging host working!

But some registers toggle if read more than one or two times (registers differ)

Reading PMU also has SPI signals??!

2.12.2014

用SPI读ABB Register 会使registers值回到复位后的值。

测试了重复读取register值，若干次内总会使register各比特的值同时回到复位状态。 读取次数是不确定的。

因此怀疑是读SPI会产生ABB 复位信号。

经实验，将ABB 复位信号改为手动控制(默认是选择Power Reset),上述现象不会出现。

因此读SPI 应该很有可能是影响了Power Reset 导致register复位。

另外实验证实SPI CLOCK toggle是导致Power Reset的直接原因(因为断掉CS DO完全不影响register复位)。

读SPI时，发现用示波器看的话是可以读的(非全零)，否则可能全零(读不出来)。加了个电容解决？！（此时用示波器看就不行了）

2.13 Find that at least 2 gpios in write could let registers reset!!!!!

Seem that after hardware reset, software will send 57, 00, 00, 00, and a0, aa, aa, aa. (Nothing)

PowerKey with AC ON(charger on) could not reset. Such as

PIN is NOT OK?! voltages？

Write with Buffering of SPI, 4.2v about. No.

2014.02.18 could not good inhabit the possibility that signal could not goes into those gpios!

Need to fix by eco! But ECO could work of course?!

Any other way to communicate with DBB?! But could TEST more!

2014/2/21

As verified, data received by pc from uart such as a1 …, … is null. As let RXD dis/connect will cause it.

But what 11 comes from ?

And why pmu_tst let debug host not work?!

2014/2/24

- ABOUT DEBUG HOST:

Debug host if needs running, to train HST_RX with SPI CS. IF 11 got, then host could do!

- Why host read bt registers always the same!

Simulation result: get the BT RESET(SYS 23, use RESET CLR to de-assert), first OC_32K(20)(in clock other), OC_26M(19)(in clock other), CLK_SYS_BT(36)(in enable2) to do.  BT enable in PMU should be opened!

Guess?!

Comments: debug host reading causes DMA access.

(现在写的是01600000, offset 0x20 寄存器0x47)

2014-02-27

(1)A good solution got when checking why simulation and chip running .

因为rom程序会判断flash起始是否是0xd9ef0045及BOOT MODES, 起始已写，且BOOT MODES满足就会执行flash程序， 否则进入到下载模式。flash起始是否是0xd9ef0045是进入flash执行的必要条件。

手动在flash中写0xd9ef0045与语句。

2014.02.28

Try to write flash with some words, failed. Why?

words to write:
J INSTRUCTION = J 81e010c4

1e010c4/4 = 780431

08780431

45

00

ef

d9

00

00

00

00

00

00

00

00

00

00

00

00

08

78

04

31

Codes example for programming:

以 地址0 为例：

1。擦除：

w 0xa1a25000,0x06   //lixr comment: written enable (06H)

w 0xa1a25000,0x00000020  //sector erase

其中，0x00000020  中  高24bit是地址，低8bit 为擦除命令，擦除4K。

2. 写：

w 0xa1a25000, 0x06 //write enable(06H)

w 0xa1a25008, 0x12  //write to FIFO

w 0xa1a25008, 0x34

w 0xa1a25008, 0x56

w 0xa1a25008, 0x78

w 0xa1a25008, 0x9a

w 0xa1a25008, 0xbc

w 0xa1a25000, 0x00000002  //page program

往 0x00 位置 写入0x12 0x34 0X56 0X78 0X9A 0XBC 6个数。

其中 0x00000002， 高24bit 是地址， 低8bit 是写命令。

=====codes=====

2014.03.04

(1)FLASH could not be written in. Finally a likely possible reason is found out. SPI_FLASH_EN_B is NOT bonded low(leave unconnected)!

In current setting(ext_mem_mode_3 is tied ‘0’). nand_sel in SPIFLASH CONTROLLER is set either by 0x24 bit 0, or SPI_FLASH_EN_B pin. To let 0x24 bit 0 cleared, nand_sel is ‘1’. So, SPI_FLASH_EN_B is high in!!!

HOW it was found:

Step1: To read data from flash, all zero.

Step2: Try to erase with 20h command, no effects.

Step3: Try to program with 02h command, no effects.

Finding: PD mode necessary to let read all 1 in step1.

After writing no effects. VIO and VMEM inconsistence is found, to adjust it by using external voltage, with internal vio18_pu down. (later find it not necessary)

With Voltage adjusted, and use codes above to erase sector and programming, no effects.

So, to setup a simulation environment good. Find a consistence between simulation and real chip. That in simulation, flash is ready, d9ef0045 ok, while chip not having this. So going to different codes, one for downloading, one for running. Finally, simulation with uart model sending setup(later in time to send, error data in uart txt appeared and fixed). And 0)find that 0x20 (sector erase)not supported, only 0x60(chip erase) ok.

Use new chip erase, at beginning, program address 00 with 00 seem works! But later tests not work. In fact, very casual, this phenomenon is not determined to be deterministic(in fact it is, found after nand_sel issue found), the reason is that we tests programming non-zero, but not zero. This should be some conclusion got, not not, though not much effectiveness on finding nand_sel issue more quickly. A lecture: But, if, conditions could be frozen, and phenomenon could be always at that condition, a good thing it is!!!

So, checking internal registers of flash controller, so as to find why simulation OK(WREN and WRITE FIFO procedure not a factor) but really not work. 1)Found nand_sel is 1 in real, but 0 is it in simulation, and 2)simulation with nand_sel set is showing a phenomenon consistent with real. Check it, SPI_FLASH_EN_B is then checked, and found not bonded(need low bonding).

Details about nand_sel set phenomenon,

2014.03.05

WHY ABB write codec not works, but it more than once worked.

As THIS SPI is shared DO between ABB and codec. SO, I checked with sch and codes for their sharing way: with ABB 1f[0] ‘0’ to select ABB do, ‘1’ to select codec do.

TRY: set ABB 1f[0] to 1. ABB REGISTERS could not be accessed(should be like this). BUT codec spi not works also.

Check that why 1f[0] before not set? things may happen that when CODEC SPI access work, some pre-setting already done, including 1f[0].

TRY: USE 8809p some setting steps, 1f[0] is set during these steps, CODEC SPI work!

CONCLUSION: Similar as not working scenario. IF WORK, needs to repeat it, and get the requirements and necessary environment setting, in case of not working next time, not having a clue for its reason.

2014.03.10

Debug Host, PMU SPI not work!

USE NEW chip for board. Except I2C is FINE, other interfaces and debug host could not work.

Most probable is GPIO not works.

CHECK this by driving clocks to UART RX. It indicates that only faster to over about 50MHZ, 0x11 could not be sent back. It shows that GPIOs not switches fast enough. (Reason: ten period @ 921600HZ, it is about 90KHZ to let HOST not sends 0x11 back).

Seen 0x11 sometimes sent back itself, set RX high to avoid a possible low on RX. AND CLOCKS on RX will stop 0x11generation about 46KHZ, almost consistent as GPIO working well.

It works! By working enough time.

2014.03.19

Bypass thing: send 0xfd to h2p_status, get 0xff000001 response, send 0xff to h2p_status, get 0xff000004 response. Though each sending and response task takes over 20 seconds.

2014.03.25

How about it not working for while, if uart running ok, but PC is bad. fet

How to act like

2014.03.27

Use BB reset from analog after power-on, it could let debug host sending 0xff. If second time, 26m is gone.

Why power-key not necessary sometime.

2014.04.02

Why iomux_abb_pmu_tst ‘1’, p05 ‘1’ will cause i2c, uart not work.

Decide: I2c is not able to write or read any more!

I tried to let other tsts, and p05 ‘0’, not affecting.

So, I check codes, only 26m is possible.

2014.04.03

P14(vmmc supply) if RU, then it will let BB not work: host not ack, reset could not reset; and may cause re-power on could not work!  Note that at first few re-power, a bigger current is shown, which needs power key down (up again not affects) to return to normal current.

P10 will let temporary BB not work.

QUITE SOME TIMES will let IO down?!

2014.05.06

Use fpga sim to do. It is found that even no pc is out. It is terrible, however, later it is turned out to be a result of missing debugging signals from _dbb. This tells me that, the attitude to treat problems or difficulties like normal is not easy. When you already have some douts before doing something, things are ticky.

Resets & clocks are probing the wrong!

2014.05.07

Use good things. pc is on! But no things.

CLOCK is terrible?! no way to think about the ucf?! it is generated by sdc! If no way to get clear about something, how will go?!

2014.05.09

A good law: try to figure out an idea or path first. First know what you could/want see/do.

2014.05.12

flash_download not ‘1’ in fpga! but forced when sim

such as HSPLIT is not well good initialized, making not returned from split, as it is unknown after un-split, too.

for good, BUS REQ, good connected!

2014.05.13

Uart is ok. Accessing spi controller is ok, gpio controller is ok. But seems running bad, and reading flash not works.

Why load memory not works?  Memory file is inverted: I use bin2assic –bin option to go!

flash not works? I new constraint one.

FLASH not connected.

2014.05.21

Check three chips, they could work.

But 11 frequency is 37Khz off, but is 59KHz computed?

AC ON let core on, but no 26MHz.

BT ENABLE sometimes let 26MHz on, sometimes not.

RESET lets reading not stable.

[37][14]???!!

r_pwrkey_on || charger_on_inn || pwrbb_inn is toggling when use only power key, stable if charger on, if charger on removed. still stable.

if charger on only, no powerup.

- AC ON 与POWERKEY要配合使用？ 一块板子是这样

- 单独AC ON不能开机，根本没有反应

- 单独POWERKEY好像也不能开机， 但是resetn_tsc不稳定

- POWERKEY后，AC ON则开机。释放POWERKEY AC ON均没有问题。释放顺序？？

- AC ON后，POWERKEY 可以开机。

发现一个问题，下列信号在单独POWERKEY  ON 时跳动

Register_37[14] :  resetn_tsc = resetn_tsc_dr? resetn_tsc_reg: ((tsc_enable|key_enable|gpadc_enable_inn)&power_on_inn);

怎么忽然都不能开机了，但是有POWERKEY按着，不管是不是AC ON着，resetn_tsc始终是变化的。

|  | AC ON | POWERKEY长按 | DBB是否开机 | I2C工作 | AC ON与PWRKEY释放后是否仍工作 |
| --- | --- | --- | --- | --- | --- |
| 芯片1 | ● | ● | ● | ● | ● |
|  | ● | ○ | ○ | ○ | - |
|  | ○ | ● | ○ | ● | - |
| 芯片2 | ● | ● | ● | ○ | ●(过一段时间不工作) |
|  | ● | ○ | ● | ○ | - |
|  | ○ | ● | ○ | ○ | - |
| 芯片3 | ● | ● | ● | ○ | ● |
|  | ● | ○ | ● | ○ | - |
|  | ○ | ● | ○ | ○ | - |
| 总结 | 1．AC ON与POWERKEY 长按同时使用总是可以DBB开机 2. 软件可以开机 3. I2C经常不能使用 |  |  |  |  |
| 下一步方向 | 1．DBB能工作，I2C却不能使用是很奇怪，这时26MHz是起振的 2．有些芯片AC ON 之外还需要POWER KEY(芯片2 3)DBB才能开机，逻辑上暂时不好说通 |  |  |  |  |

2014.05.26

Why board 6 always ok?!

why ac on could not let on?

只有0x11，没有reset.

|  | AC ON | POWERKEY长按 | DBB和是否开机 | I2C工作 | AC ON与PWRKEY释放后是否仍工作 |
| --- | --- | --- | --- | --- | --- |
| 芯片1 | ● | ● | ● | ● | ● |
|  | ● | ○ | ○ | ○ | - |
|  | ○ | ● | ○ | ● | - |
| 芯片2 | ● | ● | ● | ○ | ●(过一段时间不工作) |
|  | ● | ○ | ● | ○ | - |
|  | ○ | ● | ○ | ○ | - |
| 芯片3 | ● | ● | ● | ○ | ● |
|  | ● | ○ | ● | ○ | - |
|  | ○ | ● | ○ | ○ | - |
| 总结 | 1．AC ON与POWERKEY 长按同时使用总是可以DBB开机 2. 软件可以开机 3. I2C经常不能使用 |  |  |  |  |
| 下一步方向 | 1．DBB能工作，I2C却不能使用是很奇怪，这时26MHz是起振的 2．有些芯片AC ON 之外还需要POWER KEY(芯片2 3)DBB才能开机，逻辑上暂时不好说通 |  |  |  |  |

05.26

最初是有三块芯片host都可以用的。

不过后面出现了一些不稳定，有时可以用，或者有些板子不能用。

但是今天晚上测了一下：

三块芯片都稳定，只是其中一块芯片(芯片6)需要同时按下POWERKEY。

注意到在某些测试之后，芯片的稳定性可能是因此改善：

1.碰上不稳定的时候，把HSTRXD在外接高是能显著改善DBB开机稳定性的。这个有三块芯片都HST RXD不拉高与拉高各测了3-5次，拉高的时候都能开机。而不拉高则三块芯片都有可能开不了机(芯片6除外)。

今次三块芯片都稳定的状况是在做了上述测试之后发现的。

2. 顺便说一下上一版芯片的状况，HST RX方向(泛指对RX请求不响应)若不能工作，在HST RXD口接方波测试之后，则很大概率能使RX稳定可用。

所以，可能有某些影响稳定性的确定性原因。

By the way , 为减少不必要的影响，推荐 放电干净，HST Reopen,  重新上电的程序。

2014.05.27

画面感很重要：什么时候有个reset, 什么时候有个32k clock.

Power in on at the first, then power-key 32K is first on, this let interface work, then Power on to let other modules de-asserted resets. 26M clock will be on after DBB started.

比如: 1)为什么需要AC ON和PWRKEY同时使用呢？

check 了PWRKEY的fanout, 都和charger_on_inn或。 而这个charger_on_inn，推测这个charger_on_inn是没有起来的。 果真：charger_on_inn是需要32K的，而此时32K确是还没有开的。

解决方案，可以仿真。

2)为什么POWERKEY不能开机。 因为power_on为高即会使开机变关机。其实应该很容易发现。

解决方案。还是仿真。

一些现象被忽略，一些现象却被留意，大部分取决于对哪些现象是熟悉的。比如对HST 0x11不明所以，忽略，结果，很有可能是HST RX端被干扰。这也会使HST TX被中断传输，发0x11。

比如对有时候AC ON即可以开机，有时候需要两者同时。哪个更应该研究呢？一开始不知道。结果证明，后者是应该去研究的。反过来推断，因为它依赖更多东西。因此值得研究。依赖少，可能是因为偶然，依赖多，则很难说是偶然了。

2014.05.28

charger 被烧坏?! 怎么发现的?

为什么vcore在开机之前就有了。

1p8 of flash not connected!???!

0529 updates: FLASH PAD now use vio1p8, but default 1P8 is not on. (IO PAD use either 1P8 or 2P8, 2P8 default is on)

THIS COULD LET AN IMPROVEMENT!

pu_buck1: output of pmu interface. Vcore is it! There happens that pu_buck1 is on when neither ac on or pwrkey, meaning that pu_buck1 is on when yet not powered on.

Such as :  PON POFF ACTIVE LP

pu_buck1           = pd_mode_sel ? pu_buck1_nor           : pu_buck1_def;

//where pd_mode_sel is at register_01[0]. default 0, so pu_buck1_def is selected. Before power_on_vcore, it should be pu_buck1_poff(0), else pu_buck1_pon(1).

- pu_buck1_def          = power_on_vcore ? pu_buck1_pon          : pu_buck1_poff;

- pu_buck1_nor           = ( (~lp_mode) | dcdc1_active ) ? pu_buck1_act           : pu_buck1_lp ;  //lp_mode is controller in at register_22 and lp_mode from DBB

// dcdc1_active = xen_buf || xen_bt_buf || pu_fm || pu_bt;

dcdc2_active = xen_buf || xen_bt_buf || pu_bt;

2014.05.30

调试BT TX通路:

- 往BT APB GPIO 0x20寄存器写入0x47, to let TX ON. Some waveforms expected at BT RF.

BUT NOT, EVEN 3C registers at BT interface is all pd?!

It is supposed that if 3C register in BT DIG is to be reflecting real, STATE machine in 5408(BT dig interface) should be started by tx_on from rda5875_mips(DBB).  In other words, txon_rf would update 3C, and txon_rf is on when state_tx is not in IDLE state. This means that 3C would be updated if state_tx out of idle state. it is about tx on from dbb is whether gated or not.

find out clk for this state machine is not active! As source clock is not active open.

First PMU interface register_61[15 14] write(register_14 read is not stable), then CLOCK ON, reset de-asserted, then write registers.

2014.06.03

6.03.01

测试一个蓝牙的TX射频通路，写BT APB GPIO 0x20寄存器0x47，3C寄存器读出值表明TX ON已经传递到蓝牙芯片。但是后面发现是射频的口不对，朱诗宁来看A5 pu_ldo_on 后电流变大很多的问题发现了射频口没接对的问题。

06.04.01

AIF可以支持64fs

06.05.01

9号板被烧掉?

-----------

将gpio p1_4的ruen信号写’1’，则会导致大数字挂掉。

把p1_3, p1_1的ruen写’1’均不会挂掉；

把p1_4的rden信号写’1’,也不会刮掉，此时再把p1_4的ruen写’1’，会刮掉。

-------

06.06.01

P1<4>不能上拉操作，否则HST口挂掉。待确认：

0x28 写0xAAAAAAAA, 0x00读出的值(gpio3 2 1 0 in)是0xAAAA8AAA,    P1<5>上拉无效。  待确认：

然后再把0x30写0xAAAAAAAA，0x00读出0x80000A00, 所以P1<1><3>   P3<7>不能下拉。  正常：电路板外部接高了。

0x2C 写0xFFFFFFFF, 0x04读出的值(gpio7 6 5 4 in)是0xFFFFFFF7,    P4<3>不能上拉, 正常。 因为oenb为0

然后再把0x34写0xFFFFFFFF，0x04读出0x000E0000, 所以P6<3>< 2> <1>不能下拉.  正常: P6<1>为 HST_RXD，eco已改为固定上拉。

正常： P6<3> <2>未封装出Pin， 为I2C 口，外部上拉。

注: oenb ‘0’, gate off, 下拉; oenb ‘1’, gate off when !ruen & !rden. Otherwise it is pull up when ruen & !rden. Else is pulled down.

2014.11.26

1. why not starting running flash. Downloading is OK, but run is not！So rom should be OK, it is turned to be flash not consistent with ROM.

2. why shutting down not ok. but if watching dog is not ever opened, OK. I first think that something is lying at software about watching dog. running simulation shows that if power bb is going down, shutting down is ok. check that watching dog is opened, could restart! Lastly, it is showing that something is stucked, then wathched

3. why only charger on not ok. upper voltage, OK!

2014.12.01

1. USB could not be detected. Checking VUSB, voltage is at 4V. charge pump is not on. Set PMU 07 register bit 1 to 0(CP_BYPASS 0), work! But codec not work at it!

2014.12.02

1.fm not work always. clock out  not stable?!

BT not paired!

2014.12.10

1. bt tports only three working out of 8.

assuming that gpios are ok! but iomux_jtag forbids it!

2014.12.19

XCPU could read, BT CPU could read.

But XCPU could not write, because it checks SPI FLASH STATUS NOT READ BUSY.

HOW? IT’S A SOFTWARE PROBLEM!

As for reading, both could, as two masters, but for write and read, READ BUSY check is like an arbiter, BUT THIS IS POOL because could not wait such a not BT read idle time!

Solution: software-BT do while?(BT CPU clock gated off not works!)

2014.12.29

5855是否EDR OK？！

这是什么问题？要找的是一个版本问题？还仿真干什么？！

2014.12.31

note that s_loop not very good when AIF is on-going. AS guess AIF RXD should not affects, So should be AIF TXD affects. DISABLE STB_EN verified it!

THEN it should be about TX_STB, so I tried to adjust STB FREQ, OK!!!(at 26 MHz)

NOW, I needs to fix ok at 48MHZ.

2015.01.04

Know more about PMU states,

Know more about PLL clock generation.

2015.01.05

TX_SAMPLE和RX_SAMPLE在codec中是一起的。

2015.01.07

hp(chip)LINE(board)

vop  speaker(board)

recv -> X

spk -> X

2015.01.08

2015.1.12

As SYNC with RX is OK!

ONLY codec, 26m not worked!

2014.1.20

IOs

2014.1.21

Mic input

USB detect interrupt, INT 没有打开。

Final test documents

| 时间 | 项目 | 状态 | 成果 |
| --- | --- | --- | --- |
| 2015.1.10-2015.1.16 | 1)5855 功能:codec record/play/loop | 完成(100%) | 获得了保持录音与play一致的方法，解决了录音质量不够高的问题 |
| 2015.1.17-2015.1.23 | 1)5855平台：SCAN/BIST clock gpio ECO | 1.前端准备好(100%); 2.后端工作进行中(?) | 已更新网表并仿真 |
|  | 2)5855功能 : 红外功能测试 | 1.软件测试中(50%) | 编写完成完整的控制文档(寄存器说明等) |
|  | 3)5855 功能：USB下载 | 1.进行中(USB可枚举)(50%) | 帮助解决了USB识别的问题(软件) |
|  | 4)5855 BT RF: 接收增益和发射增益不稳定 | 1.模拟调试中(?) | 及时支持相关的接口控制 |
| 5855 2015.1.23后续事项 | 1)Final Test 方案 | 1.SCAN/BIST 方案(100%); 2.Host debug(50%) |  |
|  | 2)USB 下载， ECO， ROM更新，红外测试，BT RF调试 |  |  |

2015.1.26

FT Host 说的是怎么debug.

2015.02.09

红外，因为clock被关掉.

2015.2.10

pmu 03[8] pu_vio1p8_act为0读不到psram

2015.2.11

BUSRT MODE的问题。 从waveform中看到，仿真中有看到有ZZZZabcd的数据。此为在PSRAM输出的第二个16 bits小于一个cycle的情况下，chip未能latch到第二个16bits。实际中会latch到第1个16 bits,所以低16bits和高16bits相同。如何解决：psram自己的输出估计没法改，所以看latch的方式是否可以改，因此试了使用ddr_mode_en来获取另一个沿的数据，不过应该是因为影响到其它部分，所以失败；第2个方法是只在latch第2个16 bits的时候切换为另一个沿(BURST_MODE为0)，成功！

在芯片上，将BURST MODE改为0，可以读不镜像，但是第2个16 bits(低16 bits)不能写。因此把WRITE BURST MODE改为0，成功！

2.13

PRAM的模式问题；

BT的DMA问题。

3.4

对PLL_CTRL操作会让USB挂掉！

when at_PLL_CTRL =>

if Write_Unlock_H='1' then

regCfg_Pll(p_Pll_Enable)             <= PWDATA(l_Pll_Enable);

regCfg_Pll(p_Pll_Lock_Reset)         <= PWDATA(l_Pll_Lock_Reset);

regCfg_Pll(p_Pll_Bypass)             <= PWDATA(l_Pll_Bypass);

regCfg_Pll(p_Pll_Clk_Fast_Enable)    <= PWDATA(l_Pll_Clk_Fast_Enable);

regCfg_Pll(p_BB_Pll_Enable)          <= PWDATA(l_BB_Pll_Enable);

regCfg_Pll(p_BB_Pll_Lock_Reset)      <= PWDATA(l_BB_Pll_Lock_Reset);

regCfg_Pll(p_BB_Pll_Bypass)          <= PWDATA(l_BB_Pll_Bypass);

regCfg_Pll(p_BB_Pll_Clk_Fast_Enable) <= PWDATA(l_BB_Pll_Clk_Fast_Enable);

constant l_Pll_Enable:                          integer := 0;

constant l_Pll_Lock_Reset:                      integer := 4;

constant l_Pll_Bypass:                          integer := 8;

constant l_Pll_Clk_Fast_Enable:                 integer := 12;

constant l_BB_Pll_Enable:                       integer := 16; --Add by Lih 2011-01-01

constant l_BB_Pll_Lock_Reset:                   integer := 20;

constant l_BB_Pll_Bypass:                       integer := 24;

constant l_BB_Pll_Clk_Fast_Enable:              integer := 28;

2015.3.11

Sim, because I set BT clock on, before it is set in XCPU flash program, so in BT rom, jumps to uninitialized address!! Cause this address is loaded to ram from flash by XCPU!

2015.3.13

USB调试的问题。 因为中断的地方放了什么不该放的？！！软件架构？！！