USB has following clocks:

|  | Speed | Usage |  |  |
| --- | --- | --- | --- | --- |
| Clk_usb_pll_tx | 480M |  |  |  |
| Clk_usb_pll_rx | 480M |  |  |  |
| Clk_usb_sys | 60M |  |  |  |
| Clk_usb_hdc | 12M |  |  |  |
| Clk_usb_sys_d | 60M |  |  |  |
| I_clk_usb_480m_cdr_dft | 480M |  |  |  |

Confused. Above clocks are in clock diagrams!

From design perspective, there are following clocks

|  | Speed | Usage | Generated clock source | Source clock module | Description |
| --- | --- | --- | --- | --- | --- |
| XCLK | 60M | Transceiver clock | i_clk_usb_480m_pll | USBPHY | i_clk_usb_480m_pll is 8 divided, delayed or inverted to generate utmi_clk. |
| CLK | Same as SYS | USB core clock | - | DBB | Number PER_USBC(11) gated sys clock |
| NRST |  | USB core Reset |  |  |  |
| NRSTX |  | Transceiver Reset |  |  |  |