- rda5856 SYS CLOCK时钟简要结构：

两级clock switch。

第一级为Final switch(sys clock switch), 选择两个第二级的clock switch的输出，即clk_norm_sys 或者clk_slow_sys。

第二级的slow clock switch 选择dig 26m或者32K； norm clock switch选择DIG 26/39/52m或者pll divider output。

图1：rda5856 sys clock结构

- 基于简单原则，使用如下系统时钟源选择策略

- 正常工作模式选择clk_norm_sys。

- 睡眠模式选择clk_slow_sys。

- 32k只使用crystal output.主要考虑到若使用lpo_32k,则需要判定是否不需要高精度的32k,而这并不容易，且使用lpo_32k相比crystal 32k减少了100uA左右，不是非常显著。因此这里32k始终使用crystal output。clock_initial()函数实现此功能。

- DIG_EN和sdm_clk_test固定为1。由于两者的硬件默认值均为0，因此需要在软件初始化部分将它们改为1，并不再在任何地方改动。clock_initial()函数实现此功能。

除此处外软件应避免操作3，4相关的寄存器。

- 硬件TCU不使用，所有sysclk流程完全依赖以下函数。clock_initial()函数会将tcu模块clock gate。除此处之外软件应避免操作任何TCU相关寄存器。

- 约定流程与函数

函数1：sysclk_initial()

{

/////////// set sys clock at dig_26m

hwp_sysCtrl->REG_DBG = SYS_CTRL_PROTECT_UNLOCK;

//固定策略4：use DIG_EN

hwp_sysCtrl->Sel_Clock |= SYS_CTRL_DIGEN_H_ENABLE;  //Sel_Clock: Here must use two steps, not combined with the next line

hwp_sysCtrl->Cfg_Clk_Sys =  SYS_CTRL_FREQ_26M;

| SYS_CTRL_FORCE_DIV_UPDATE;

hwp_sysCtrl->Sel_Clock &= ~ SYS_CTRL_SYS_SEL_FAST_SLOW; //Sel_Clock: Here must use two steps, not combined with the above line

///////固定策略5：NOT USE TCU. RESET TCU here, Gate tcu clock here

hwp_sysCtrl->REG_SYS_CTRL_CLK_OTHER_DISABLE |= SYS_CTRL_DISABLE_OC_LPS; // (Correct SYS_CTRL_DISABLE_OC_LPS definition to 1<<11!)

// active 26m pu/pd only be controlled by TCU_LPS_PU_CLK_RF_ON(见附图1，auto control is disabled)

hwp_sysCtrl->REG_DBG = SYS_CTRL_PROTECT_LOCK;

//////////初始化sysclk的约定选择(dig_en已设置)

//固定策略4：Use pll_aux_sdm_clk_test_en_en

ABB/aux_pllsetting1 | = pll_aux_sdm_clk_test_en_en;  // pll_aux_sdm_clk_test_en_en = (1<<1)

//固定策略3：use 32k from cystal 26m， and enable 6m5

PMU: 0x64[15,14] = ‘b10;  //select 32k from crystal 26m

PMU: 0x61[4,3] = ‘b10;  //enable 6m5, no issue here. lp_mode 26m crystal disabled when active 26m enabled, auto enabled when active 26m disabled

}

附图：

函数2： sysclk_request(Bool Sleep, Bool Wakeup, xx new_freq)

{

if (wakeup)

{

sysclk_wakeup();

}

else if (sleep)

{

sysclk_sleep();

}

else

sysclk_change(freq);

}

函数3：sysclk_wakeup()

{

new_freq = hwp_sysCtrl->Cfg_Clk_Sys & SYS_CTRL_FREQ_MASK;

//Enable active 26M

hwp_tcu->LPS_PU_Ctrl  &=  ~TCU_LPS_PU_CLK_RF_ON;

while(! (hwp_sysCtrl->Sel_Clock & SYS_CTRL_RF_DETECTED_OK));

if(new_freq > 52M)

{

pu_bbpll();

}

else if (new_freq == 52M or new_freq == 39M)

{

//Enable Dig52M

PMU: 0x66[0]写为1

}

hwp_sysCtrl->REG_DBG = SYS_CTRL_PROTECT_UNLOCK;

//switch to norm clock

hwp_sysCtrl->Sel_Clock &= ~ SYS_CTRL_SYS_SEL_FAST_SLOW;

//slow clock switch to 26m

hwp_sysCtrl->Sel_Clock  &= ~SYS_CTRL_SLOW_SEL_RF_OSCILLATOR;

hwp_sysCtrl->REG_DBG = SYS_CTRL_PROTECT_LOCK;

}

函数4：sysclk_sleep()

{

cur_freq = hwp_sysCtrl->Cfg_Clk_Sys & SYS_CTRL_FREQ_MASK;

hwp_sysCtrl->REG_DBG = SYS_CTRL_PROTECT_UNLOCK;

//slow clock switch to 32K

hwp_sysCtrl->Sel_Clock  |= SYS_CTRL_SLOW_SEL_RF_OSCILLATOR;

//switch to slow clock

hwp_sysCtrl->Sel_Clock  |=  SYS_CTRL_SYS_SEL_FAST_SLOW;

hwp_sysCtrl->REG_DBG = SYS_CTRL_PROTECT_LOCK;

if (cur_freq > 52M)

{

pd_bbpll();

}

if (cur_freq == 52M or cur_freq == 39M)

{

//disable 52m

PMU: 0x66[0]写为0

}

//disable active 26m

hwp_tcu->LPS_PU_Ctrl  |=  TCU_LPS_PU_CLK_RF_ON;

}

函数5：sysclk_change(xx freq)

{

cur_freq = hwp_sysCtrl->Cfg_Clk_Sys & SYS_CTRL_FREQ_MASK;

new_freq = freq;

if (new_freq == cur_freq)

return;

if(new_freq > MAX_FREQ(104M))

return;

if (cur_freq == 26M)  //dig 26m to other inputs

{

////////toggle inactive switch inputs for change

//enable 52m

PMU: 0x66[0]写为1

//pu pll

pu_bbpll();

////////switch to new input

hwp_sysCtrl->Cfg_Clk_Sys =  SYS_CTRL_FREQ_xx   //

| SYS_CTRL_FORCE_DIV_UPDATE;

////////switching off not selected switch inputs

if(new_freq == 39M or new_freq ==52M)  //dig26m to dig52m class

{

pd_bbpll();

}

else  //dig26m to pll class

{

//disable 52m

PMU: 0x66[0]写为0

}

}

else if (cur_freq == 39M or cur_freq == 52M) //dig52m class to others

{

////////toggle inactive switch inputs for change

pu_bbpll();

////////switch to new input

hwp_sysCtrl->Cfg_Clk_Sys =  SYS_CTRL_FREQ_xx

| SYS_CTRL_FORCE_DIV_UPDATE;

/////switching off not selected switch inputs

if (new_freq == 26M or new_freq == 39M or new_freq == 52M ) //dig to dig

{

pd_bbpll();

if(new_freq == 26M) // dig52m class to dig 26m

{

//disable 52m

PMU: 0x66[0]写为0

}

}

else	// dig52m class to pll

{

//disable 52m

PMU: 0x66[0]写为0

}

}

else if (cur_freq > 52M)

{

if(new_freq > 52M)  //pll to pll

{

////////switch to new freq, but the same input

hwp_sysCtrl->Cfg_Clk_Sys =  SYS_CTRL_FREQ_xx   //

| SYS_CTRL_FORCE_DIV_UPDATE;

}

else  //pll to dig

{

////////toggle inactive switch inputs for change

//enable 52m

PMU: 0x66[0]写为1

////////switch to new input

hwp_sysCtrl->Cfg_Clk_Sys =  SYS_CTRL_FREQ_xx   //

| SYS_CTRL_FORCE_DIV_UPDATE;

/////switching off not selected switch inputs

pd_bbpll();

if (new_freq == 26M) //pll to dig26m class

{

//disable 52m

PMU: 0x66[0]写为0

}

}

}

}

函数6:  pu_bbpll()

{

hwp_sysCtrl->REG_DBG = SYS_CTRL_PROTECT_UNLOCK;

hwp_sysCtrl->Pll_Ctrl   =     SYS_CTRL_PLL_ENABLE_ENABLE

| SYS_CTRL_PLL_BYPASS_PASS

| SYS_CTRL_PLL_CLK_FAST_ENABLE_ENABLE

| SYS_CTRL_PLL_LOCK_RESET _RESET;

//check pll lock

hwp_sysCtrl->Pll_Ctrl   |= SYS_CTRL_PLL_LOCK_RESET _NO_RESET;

hwp_sysCtrl->REG_DBG = SYS_CTRL_PROTECT_LOCK;

while(!(hwp_sysCtrl-> Sel_Clock & SYS_CTRL_BB_PLL_LOCK_RESET_MASK));

}

函数7：pd_bbpll()

{

hwp_sysCtrl->REG_DBG = SYS_CTRL_PROTECT_UNLOCK;

hwp_sysCtrl->Pll_Ctrl   =     SYS_CTRL_PLL_ENABLE_POWER_DOWN

| SYS_CTRL_PLL_BYPASS_BYPASS

| SYS_CTRL_PLL_CLK_FAST_ENABLE_DISABLE

| SYS_CTRL_PLL_LOCK_RESET _RESET;

//check pll unlock

hwp_sysCtrl->Pll_Ctrl   |= SYS_CTRL_PLL_LOCK_RESET _NO_RESET;

hwp_sysCtrl->REG_DBG = SYS_CTRL_PROTECT_LOCK;

while(hwp_sysCtrl-> Sel_Clock & SYS_CTRL_BB_PLL_LOCK_RESET_MASK);

}